Rohde & Schwarz Signal Generator SMY

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Rohde & Schwarz Signal Generator SMY.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Rohde & Schwarz Signal Generator SMY

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                        Function Name:

Initialize                            rssmy_init

Application Functions

Output Waveform                    rssmy_outputWaveform

Configuration

Basic Settings

RF Frequency Setting            rssmy_setRF

RF Level Setting                rssmy_setRFLevel

Audio Frequency Setting         rssmy_setAF

Special Settings                rssmy_specialSetting

Modulation Settings

Amplitude Modulation Setting    rssmy_setAMMod

Frequency Modulation Setting    rssmy_setFMMod

Pulse Modulation Setting        rssmy_setPMMod

Phase Modulation Setting        rssmy_setPhMMod

Variation Settings

Freq./Level Steps               rssmy_setFreqLevelStep

Modulation Steps                rssmy_setModulationStep

Action Functions

RF Output                          rssmy_rfOut

AF Output                          rssmy_afOut

Variation Step                     rssmy_singleVarStep

Utility Functions

Write To Instrument                rssmy_writeInstrData

Read Instrument Data               rssmy_readInstrData

Reset                              rssmy_reset

Self-Test                          rssmy_self_test

Error Query                        rssmy_error_query

Error Message                      rssmy_error_message

Revision Query                     rssmy_revision_query

Diagnostics                        rssmy_diagnostics

Check For Status                   rssmy_checkStatus

Close                                 rssmy_close

________________________________________________________________________________

Rohde & Schwarz Signal Generator SMY

This instrument module provides programming support for the Rohde &

Schwarz Signal Generator SMY 02. The module is divided into the following

functions:

Functions/Classes:

(1) Initialize:

This function must be executed first! Initializes the SMY on the GPIB.

(2) Application Functions: (Class)

This class contains high-level examples.

(3) Configuration: (Class)

This class ontains classes and functions for the configuration and setup

of waveform and other instrument functions.

(4) Action Function: (Class)

This class contains functions forcing the instrument to perform an

operation - enabling and disabling RF and AF Output and step execution.

(5) Utility Functions: (Class)

This class of functions provides basic utilities for operating the

instrument.

(6) Close:

Closes the instrument.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

rssmy_afOut

ViStatus rssmy_afOut (ViSession instrumentHandle, ViBoolean AFOutput);

Purpose

Enables or disables the AF Output of the Generator. These actions are

only required if the AF output (connector AF INT [17]) is to be

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

AFOutput

Variable Type       ViBoolean

Turns AF output off/on. These actions are only required if the AF

output (connector AF INT [17]) is to be used.

Valid Range:

VI_TRUE  (1) - On (Default Value)

VI_FALSE (0) - Off

Display: In the ON state the RF LEVEL value appears in the amplitude

display, in the OFF state <OFF> appears.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (AF Output) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_checkStatus

ViStatus rssmy_checkStatus (ViSession instrumentHandle,

ViBoolean checkForStatus);

Purpose

This function specifies whether to check for an error after each function

call or not.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

checkForStatus

Variable Type       ViBoolean

Specifies whether to check for the instrument error status after the

each function call. This test takes less than 2 seconds because of

very low speed of the instrument's error reporting operations.

Valid Range:

VI_TRUE  (1) - Yes (Default Value)

VI_FALSE (0) - No

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Check For Status) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_close

ViStatus rssmy_close (ViSession instrumentHandle);

Purpose

Closes the instrument and takes it off line.

Note: The instrument must be reinitialized to use it again.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_diagnostics

ViStatus rssmy_diagnostics (ViSession instrumentHandle,

ViInt32 testpoint_numb, ViBoolean testState,

ViPReal64 reading);

Purpose

Turns test routine ON/OFF, selects the testpoint and returns the test

voltage value.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

testpoint_numb

Variable Type       ViInt32

Selects a test point number

Valid Range: 1 to 17

Default Value: 1

Nr.     Designation                  Rated value

1     All control loops ok        4.5 V +- 0.5 V

Control loop(s) failed      0 V +- 0.5 V

2     OCXO nom. temperature       4.5 V +- 0.5 V

Oven cold                   0 V +- 0.5 V

3     Tuning voltage of

reference  loop             1.8 to 13.1 V

4     Tuning voltage of

VCXO loop                   2.5 to 13.3 V

5     Tuning voltage of

RF loop                     1.5 to 22.2 V

6     10 MHz reference level      1.8 V +- 0.5 V

7     Oscillator output level     0.2 to 0.7 V

8     REF INT, OCXO not fitted    4.5 V +- 0.5 V

REF EXT, OCXO not fitted    0.6 V +- 0.2 V

OCXO fitted                 0 V +- 0.2 V

9     Collector voltage of

output amplifier V400       18.1 +- 1 V

10    Collector voltage of

output amplifier V390       14 +- 1 V

11    Collector voltage of

output amplifier V380       14 +- 1 V

12    Amplitude control

voltage 0.1 to 13 dBm      -2.5 to - 0.5 V

13    Actual value of level

control 13 dBm, RF > 8 MHz  2.6 V +- 0.5 V

14    Actual value of level

control 13 dBm, RF < 8 MHz  2.6 V +- 0.5 V

15    Divider output level        0.05 to 0.3 V

16    Nominal value of

level control 13 dBm       -2.6 +- 0.5 V

17    Peak value of external

modulation voltage

1 Vrms, AF 1kHz           1.41 V +- 0.01 V

Display: If the test routine in on, the voltage value of the test

point is output in the left-hand section of the amplitude display,

the number of the test point is output in the right-hand section.

testState

Variable Type       ViBoolean

Switches the test routine on or off.

Valid Range:

VI_TRUE  (1) - On

VI_FALSE (0) - Off (Default Value)

Display: If the test routine in on, the voltage value of the test

point is output in the left hand section of the amplitude display,

the number of the test point is output in the right-hand section.

reading

Variable Type       ViReal64 (passed by reference)

Returns a value of the selected test point.

Valid Range: Dependent upon the number of the test point selected,

see control Testpoint # (1..17).

Display: If the test routine in on, the voltage value of the test

point is output in the left-hand section of the amplitude display,

the number of the test point is output in the right-hand section.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Testpoint Numb) out of range.

BFFC0003  Parameter 3 (Test State) out of range.

BFFC0004  Parameter 4 (Reading) null pointer.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_error_message

ViStatus rssmy_error_message (ViSession instrumentHandle,

ViStatus statusCode,

ViChar _VI_FAR message[]);

Purpose

This function takes the Status Code returned by the instrument driver

functions, interprets it and returns it as a user readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  VI_NULL

statusCode

Variable Type       ViStatus

This control accepts the Status Code returned from the instrument

driver functions.

Default Value:

0 - VI_SUCCESS

message

Variable Type       ViChar[]

This control returns the interpreted Status Code as a user readable

message string.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

3FFF0085  VI_WARN_UNKNOWN_STATUS

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_error_query

ViStatus rssmy_error_query (ViSession instrumentHandle,

ViPInt32 errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function queries the instrument for an error and returns the

response.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

This control returns the error code read from the instrument's error

queue.

Note:

(1) If this control returns -1 instrument specific error was detect.

In error message are all error codes which instrument returns. This

can contain several numeric values, they are separated from one

another by means of commas (,).

errorMessage

Variable Type       ViChar[]

This control returns the error message string read from the

instrument's error message queue.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

(2) If the error code returns -1, this control returns all error

codes, they are separated from one another by means of commas (,).

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_init

ViStatus rssmy_init (ViRsrc resourceName, ViBoolean IDQuery,

ViBoolean resetDevice,

ViPSession instrumentHandle);

Purpose

This function performs the following initialization actions:

- Opens a session to the Default Resource Manager resource and a session

to the specified device using the interface and address specified in the

Resource_Name control.

- Performs an identification query on the Instrument.

- Resets the instrument to a known state.

- Sends initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

- Returns an Instrument Handle which is used to differentiate between

different sessions of this instrument driver.

- Each time this function is invoked a Unique Session is opened.  It is

possible to have more than one session open for the same resource.

Parameter List

resourceName

Variable Type       ViRsrc

This control specifies the interface and address of the device that

is to be initialized (Instrument Descriptor). The exact grammar to be

used in this control is shown in the note below.

Default Value:  "GPIB::8"

Notes:

(1) Based on the Instrument Descriptor, this operation establishes a

communication session with a device.  The grammar for the Instrument

Descriptor is shown below.  Optional parameters are shown in square

brackets ([]).

Interface   Grammar

------------------------------------------------------

GPIB        GPIB[board]::primary address[::secondary address]

[::INSTR]

The GPIB keyword is used with GPIB instruments.

The default value for optional parameters are shown below.

Optional Parameter          Default Value

-----------------------------------------

board                       0

secondary address           none - 31

IDQuery

Variable Type       ViBoolean

Specifies if an ID Query is sent to the instrument during the

initialization procedure.

Valid Range:

VI_TRUE  (1) - Do Query (Default Value)

VI_FALSE (0) - Skip Query

Notes:

(1) Under normal circumstances the ID Query insures that the

instrument initialized over the GPIB is the type supported by this

driver.  However circumstances may arise where it is undesirable to

send an ID Query to the instrument.  In those cases; set this control

to "Skip Query" nd this function will initialize the GPIB and the

Command arrays in the driver, without doing an ID Query.

resetDevice

Variable Type       ViBoolean

Specifies if the instrument is to be reset to its power-on settings

during the initialization procedure.

Valid Range:

VI_TRUE  (1) - Reset Device (Default Value)

VI_FALSE (0) - Don't Reset

Notes:

(1) If you do not want the instrument reset set this control to

"Don't Reset" while initializing the instrument.

instrumentHandle

Variable Type       ViSession (passed by reference)

This control returns an Instrument Handle that is used in all

subsequent function calls to differentiate between different sessions

of this instrument driver.

Notes:

(1) Each time this function is invoked a Unique Session is opened.

It is possible to have more than one session open for the same

resource.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0001  Parameter 1 out of range. (String not range checked)

BFFC0002  Parameter 2 (ID Query) out of range.

BFFC0003  Parameter 3 (Reset Device) out of range.

BFFC0011  Instrument returned invalid response to ID Query

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_outputWaveform

ViStatus rssmy_outputWaveform (ViSession instrumentHandle,

ViReal64 radioFreq_Hz,

ViReal64 audioFreq_Hz,

ViReal64 RFLevel_dBm,

ViBoolean RFLevelUnits,

ViBoolean RFLevelType,

ViBoolean RFOutput, ViBoolean AFOutput);

Purpose

This function is the high-level example of the use of this driver's

functions. It allows you to configure and output the waveforms.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

radioFreq_Hz

Variable Type       ViReal64

Specifies the frequency value of the RF output in Hz.

Valid Range:    9 kHz to 1040 MHz (SMY 01)

9 kHz to 2080 MHz (SMY 02)

Default Value:  100 MHz

Resolution:      1 Hz

Display:        The RF output frequency appears in

the frequency display.

Associated

Instructions:   Int./ext. reference frequency

audioFreq_Hz

Variable Type       ViReal64

Specifies the frequency value of the AF in Hz. The internal AF

oscillator is automatically switched on an as the modulations source

in the case of internal modulation. It can also be switched on for

external use of the AF signal if no internal modulation is on.

Valid Range:    0.1 Hz - 500 kHz

Resolution:    0.1 Hz 3-digit display

Display:       The modulation display indicates the AF.

Associated

Instructions:   Modulation (AM, FM, phM) two-tone Modulation

RFLevel_dBm

Variable Type       ViReal64

Specifies the level value of the RF output together with the control

RF Level Units.

Valid Range:    -140 to 13 dBm (0.03 �V to 1 V)

(settable up to 18 dBm without

guarantee of rated specifications)

Resolution:     0.1 dB

Display:        The RF Level output appears in

the  amplitude display.

Associated

Instructions:   Non-interrupting level setting

Level EMF

RFLevelUnits

Variable Type       ViBoolean

Specifies the dimension of the RF Level.

Valid Range:

VI_TRUE  (1) - Volts (Default Value)

VI_FALSE (0) - dBm

Display: The RF output level dimension appears in the amplitude

display.

RFLevelType

Variable Type       ViBoolean

Specifies whether the RF level will be set as normal or as the EMF

level.

Valid Range:

VI_TRUE  (1) - EMF

VI_FALSE (0) - Normal (Default Value)

RFOutput

Variable Type       ViBoolean

Turns RF output off/on.

Valid Range:

VI_TRUE  (1) - On (Default Value)

VI_FALSE (0) - Off

Display: In the ON state the RF LEVEL value appears in the amplitude

display, in the OFF state <OFF> appears.

AFOutput

Variable Type       ViBoolean

Turns AF output off/on. These actions are only required if the AF

output (connector AF INT [17]) is to be used.

Valid Range:

VI_TRUE  (1) - On (Default Value)

VI_FALSE (0) - Off

Display: In the ON state the RF LEVEL value appears in the amplitude

display, in the OFF state <OFF> appears.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Radio Freq) out of range.

BFFC0003  Parameter 3 (Audio Freq) out of range.

BFFC0004  Parameter 4 (RF Level) out of range.

BFFC0005  Parameter 5 (RF Level Units) out of range.

BFFC0006  Parameter 6 (RF Level Type) out of range.

BFFC0007  Parameter 7 (RF Output) out of range.

BFFC0008  Parameter 8 (AF Output) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_readInstrData

ViStatus rssmy_readInstrData (ViSession instrumentHandle,

ViInt32 numberBytesToRead,

ViChar _VI_FAR readBuffer[],

ViPInt32 numBytesRead);

Purpose

This function reads data from the instrument's output buffer and returns

it to the specified variable in memory.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) If valid data is not available at the instrument's output buffer when

this function is called the instrument will hang up and the function will

not return until it times out.  If the time-out is disabled this function

will hang indefinitely and it may be necessary to reboot the computer to

break out.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberBytesToRead

Variable Type       ViInt32

The number of bytes specified by this control should be greater than

or equal to the number of bytes which are to be read from the

instrument. If the actual number of bytes to be read is greater than

the number this control specifies then multiple reads will be

required to empty the instrument's output buffer.

If the instrument's output buffer is not emptied the instrument may

return invalid data when future reads are performed.

Default Value:  50 (See NOTE 2)

Notes:

(1) If NO DATA is available at the instrument's output buffer when

this function is called the instrument will hang up and the function

will not return until it times out.  If the time-out is disabled,

this function will hang indefinitely and it may be necessary to

reboot the computer.

(2) If the number of bytes expected is greater than 50 the value of

this control may be increased. If the actual number of bytes read is

larger than the declared size of the read buffer a run-time error

will be generated.

readBuffer

Variable Type       ViChar[]

The incoming data from the instrument is placed into this variable.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) This function does not overwrite any old data left in the string

variable from the last time the function was called. String data in

LabWindows/CVI is terminated with an ASCII null(0x0) and string

manipulation functions will only recognize data before the ASCII

null.

(3) The declared size of the string variable must be greater than the

actual number of bytes read from the instrument if it is not a

run-time error will be generated.

numBytesRead

Variable Type       ViInt32 (passed by reference)

This variable contains the actual number of bytes read from the

instrument. This is the value which is returned by the read function.

Notes:

(1) If the actual number of bytes read is less than the number of

bytes specified in the Number Bytes To Read control then the output

buffer has probably been emptied. If the read function fails and the

number of bytes read is 0, the most probable cause for the failure is

there was no data available at the instrument's output buffer.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_reset

ViStatus rssmy_reset (ViSession instrumentHandle);

Purpose

This function performs the reset of the generator.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_revision_query

ViStatus rssmy_revision_query (ViSession instrumentHandle,

ViChar _VI_FAR instrumentDriverRevision[],

ViChar _VI_FAR firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware, and tells the user with which  instrument firmware

this revision of the driver is compatible.

This instrument driver's Revision Number is "Rev 2.0, 07/1999, CVI 5.0.1"

and is compatible with Instrument Firmware Revision "1.10".

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

This control returns the Instrument Driver Software Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

firmwareRevision

Variable Type       ViChar[]

This control returns the Instrument Firmware Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_rfOut

ViStatus rssmy_rfOut (ViSession instrumentHandle, ViBoolean RFOutput);

Purpose

Enables or disables the RF Output of the Generator.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

RFOutput

Variable Type       ViBoolean

Turns RF output off/on.

Valid Range:

VI_TRUE  (1) - On (Default Value)

VI_FALSE (0) - Off

Display: In the ON state the RF LEVEL value appears in the amplitude

display, in the OFF state <OFF> appears.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (RF Output) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_self_test

ViStatus rssmy_self_test (ViSession instrumentHandle,

ViPInt16 selfTestResult,

ViChar _VI_FAR selfTestMessage[]);

Purpose

This function initiates a self test and transfers the results of the self

tests to an integer number.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

selfTestResult

Variable Type       ViInt16 (passed by reference)

This control contains the value returned from the instrument self

test.  Zero means success.  For any other code, see the device's

operator's manual.

selfTestMessage

Variable Type       ViChar[]

This control contains the string returned from the self test. See the

device's operation manual for an explanation of the string's

contents.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_setAF

ViStatus rssmy_setAF (ViSession instrumentHandle, ViReal64 AF_Hz);

Purpose

This function sets the audio frequency of the generated signal.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

AF_Hz

Variable Type       ViReal64

Specifies the frequency value of the AF in Hz. The internal AF

oscillator is automatically switched on an as the modulations source

in the case of internal modulation. It can also be switched on for

external use of the AF signal if no internal modulation is on.

Valid Range:    0.1 Hz - 500 kHz

Resolution:    0.1 Hz

3-digit display

Display:       The modulation display indicates

the AF.

Associated

Instructions:   Modulation (AM, FM, phM)

two-tone Modulation

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (AF) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_setAMMod

ViStatus rssmy_setAMMod (ViSession instrumentHandle,

ViInt32 ampModulationType, ViReal64 AMDepth_);

Purpose

Specifies the amplitude modulation parameters.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

ampModulationType

Variable Type       ViInt32

Switches AM on or off and selects modulation sources.

Valid Range:

0 - AM Off

1 - Internal AM

2 - External AM - AC

3 - External AM - DC

4 - Two-Tone AM (Int.+Ext.) - AC

5 - Two-Tone AM (Int.+Ext.) - DC

Default Value:  0 - AM Off

AMDepth_

Variable Type       ViReal64

Specifies the value of the AM depth.

Valid Range:    0 to 100 %

Default Value:  30 %

Resolution:     0.1 %

Display: The AM depth is output in the modulation display.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Amp Modulation Type) out of range.

BFFC0003  Parameter 3 (AM Depth) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_setFMMod

ViStatus rssmy_setFMMod (ViSession instrumentHandle,

ViInt32 freqModulationType,

ViReal64 FMDeviation);

Purpose

Specifies the frequency modulation parameters.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

freqModulationType

Variable Type       ViInt32

Switches FM on or off and selects modulation sources.

Valid Range:

0 - FM Off

1 - Internal FM

2 - External FM - AC

3 - External FM - DC

4 - Two-Tone FM (Int.+Ext.) - AC

5 - Two-Tone FM (Int.+Ext.) - DC

Default Value:  0 - FM Off

FMDeviation

Variable Type       ViReal64

Specifies the value of the FM deviation in Hz.

Valid Range: 0 to 20 MHz, depending on the carrier frequency.

Resolution:     10 Hz to 100 kHz, depending on the

deviation range.

Display:        The FM deviation is output in the

modulation display (3 digits).

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Freq Modulation Type) out of range.

BFFC0003  Parameter 3 (FM Deviation) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_setFreqLevelStep

ViStatus rssmy_setFreqLevelStep (ViSession instrumentHandle,

ViReal64 RFStep_Hz, ViReal64 AFStep_Hz,

ViReal64 RFLevelStep_dB);

Purpose

Specifies the RF, AF and RF Level variation steps sizes.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

RFStep_Hz

Variable Type       ViReal64

Specifies the step size of the RF frequency in Hz.

Valid Range: 1 Hz to 100.0 MHz.

Default Value:  1MHz

Resolution:     1 Hz

Display:        A step size is output in the

frequency display by pressing the

Step key in the parameter keypad

with the parameter RF set (Led

on).

AFStep_Hz

Variable Type       ViReal64

Specifies the step size of the AF frequency in Hz.

Valid Range: 0.1 Hz to 1.0 kHz

Resolution:     100 Hz

Display:        A step size is output in the

modulation display by pressing

the Step key in the parameter

keypad with the parameter AF set

(Led on).

RFLevelStep_dB

Variable Type       ViReal64

Specifies the step size of the RF level in dB.

Valid Range: 0.1 dB to 125.0 dB

Resolution:     0.1 dB

Display:        A step size is output in the

amplitude display by pressing the

Step key in the parameter keypad

with the parameter RF Level set

(Led on).

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (RF Step) out of range.

BFFC0003  Parameter 3 (AF Step) out of range.

BFFC0004  Parameter 4 (RF Level Step) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_setModulationStep

ViStatus rssmy_setModulationStep (ViSession instrumentHandle,

ViReal64 AMStep_, ViReal64 FMStep_Hz,

ViReal64 phaseModStep_Rad);

Purpose

Specifies the AM depth and FM and Phase Modulation duration variation

steps sizes.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

AMStep_

Variable Type       ViReal64

Specifies the step size of the modulation depth in %.

Valid Range: 0.1 % to 100.0 %

Resolution:     1 %

Display:        A step size is output in the

modulation display by pressing the

Step key in the parameter keypad

with the parameter AM set

(Led on).

FMStep_Hz

Variable Type       ViReal64

Specifies the step size of the deviation (FM) in Hz.

Valid Range: 10 Hz to 19.9 MHz, depending on the carrier frequency.

Resolution:     10 Hz

Display:        A step size is output in the

modulation display by pressing the

Step key in the parameter keypad

with the parameter FM set

(Led on).

phaseModStep_Rad

Variable Type       ViReal64

Specifies the step size of the deviation (phM) in rads.

Valid Range: 0.001 to 39.9 rad, depending on

the carrier frequency.

Resolution:     0.001 rad

Display:        A step size is output in the

modulation display by pressing the

Step key in the parameter keypad

with the parameter phM set

(Led on).

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (AM Step) out of range.

BFFC0003  Parameter 3 (FM Step) out of range.

BFFC0004  Parameter 4 (Phase Mod Step) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_setPhMMod

ViStatus rssmy_setPhMMod (ViSession instrumentHandle,

ViInt32 phaseModulType,

ViReal64 phaseModulDeviation_Rad);

Purpose

Specifies the phase modulation parameters.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

phaseModulType

Variable Type       ViInt32

Switches phase modulation (phM) on or off and selects the modulation

sources.

Valid Range:

0 - phM Off

1 - Internal phM

2 - External phM

3 - Two-Tone phM (Int.+Ext.)

Default Value:  0 - phM Off

phaseModulDeviation_Rad

Variable Type       ViReal64

Specifies the value of the phM deviation in radians.

Valid Range: 0 to 400 rad, depending on the carrier frequency.

Default Value:  1 rad

Resolution:     0.001 to 1 rad, depending on the

deviation range.

Display:        The phM deviation is output in the

modulation display (3 digits).

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Phase Modul Type) out of range.

BFFC0003  Parameter 3 (Phase Modul Deviation) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_setPMMod

ViStatus rssmy_setPMMod (ViSession instrumentHandle,

ViInt32 pulseModulType);

Purpose

Specifies the pulse modulation parameters.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

pulseModulType

Variable Type       ViInt32

Switches PM on or off and selects the BLANK polarity.

Valid Range:

0 - PM Off

1 - Normal PM

2 - Inverted PM

Default Value:  0 - PM Off

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Pulse Modul Type) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_setRF

ViStatus rssmy_setRF (ViSession instrumentHandle, ViReal64 RF_Hz);

Purpose

Sets frequency value for the RF output.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

RF_Hz

Variable Type       ViReal64

Specifies the frequency value of the RF output in Hz.

Valid Range:    9 kHz to 1040 MHz (SMY 01)

9 kHz to 2080 MHz (SMY 02)

Default Value:  100 MHz

Resolution:      1 Hz

Display:        The RF output frequency appears in

the frequency display.

Associated

Instructions:   Int./ext. reference frequency

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (RF) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_setRFLevel

ViStatus rssmy_setRFLevel (ViSession instrumentHandle, ViReal64 RFLevel,

ViBoolean RFLevelUnits,

ViBoolean RFLevelType);

Purpose

Sets RF Level value for the RF output.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

RFLevel

Variable Type       ViReal64

Specifies the level value of the RF output together with the control

RF Level Units.

Valid Range:    -140 to 13 dBm  (0.03 �V to 1 V)

(settable up to 19 dBm without

guarantee of rated specifications)

-134 to 19 dBm (0.05 �V to 2V )with option

SMY-B40 (settable as from -140 dBm up to 25 dBm

(0.03 �V to 4V) with restriced data)

Resolution:     0.1 dB

Display:        The RF Level output appears in

the  amplitude display.

Associated

Instructions:   Non-interrupting level setting

Level EMF

RFLevelUnits

Variable Type       ViBoolean

Specifies the dimension of the RF Level.

Valid Range:

VI_TRUE  (1) - Volts

VI_FALSE (0) - dBm (Default Value)

Display: The RF output level dimension appears in the amplitude

display.

RFLevelType

Variable Type       ViBoolean

Specifies whether the RF level will be set as normal or as the EMF

level.

Valid Range:

VI_TRUE  (1) - EMF

VI_FALSE (0) - Normal (Default Value)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (RF Level) out of range.

BFFC0003  Parameter 3 (RF Level Units) out of range.

BFFC0004  Parameter 4 (RF Level Type) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_singleVarStep

ViStatus rssmy_singleVarStep (ViSession instrumentHandle,

ViInt32 parameter, ViBoolean direction);

Purpose

Performs a variation step for the specified parameter in selected

direction.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

parameter

Variable Type       ViInt32

Selects the parameter to be changed with the set step size.

Valid Range:

0 - AF

1 - RF

2 - RF Level

3 - AM

4 - FM

5 - Phase Mod

Default Value: 0 - AF

Display: The LED of the selected Function in the parameter keypad is

on.

Associated Instructions: Frequency/Level Steps Modulation Steps

direction

Variable Type       ViBoolean

Selects the direction of the step variation.

Valid Range:

VI_TRUE  (1) - Up

VI_FALSE (0) - Down (Default Value)

Display: The LED of the selected Function in the parameter keypad is

on.

Associated Instructions: Frequency/Level Steps Modulation Steps

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Parameter) out of range.

BFFC0003  Parameter 3 (Direction) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_specialSetting

ViStatus rssmy_specialSetting (ViSession instrumentHandle,

ViBoolean refFreq,

ViBoolean nonInterruptingLevelSetting);

Purpose

Specifies the reference frequency and the non-interrupting level setting.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

refFreq

Variable Type       ViBoolean

Selects the reference frequency source.

Valid Range:

VI_TRUE  (1) - Internal (Default Value)

VI_FALSE (0) - External

Display: The text "EXT REF" appears in the frequency display if the

external

nonInterruptingLevelSetting

Variable Type       ViBoolean

An electronic attenuation setting is used over a dynamic range of 10

dB instead of the level interrupting mechanical attenuator.

Valid Range:

VI_TRUE  (1) - On

VI_FALSE (0) - Off (Default Value)

Notes:

(1) An electronic attenuation setting is used over a dynamic range of

10 dB instead of the level-interrupting mechanical attenuator. The 10

dB non-interrupting range extends from the level set when this

function is switched on to 10 dB below. Setting of a level outside

the 10 dB range is made using the mechanical attenuator set. Starting

at this new level, further level settings are made non-interruptive

again in the range 0 to -10 dB. Specifications concerning level

error, modulation depth error and distorsion factor with AM do not

apply with this function switched on.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Ref Freq) out of range.

BFFC0003  Parameter 3 (Non-Interrupting Level Setting) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC09F0  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

rssmy_writeInstrData

ViStatus rssmy_writeInstrData (ViSession instrumentHandle,

ViString writeBuffer);

Purpose

This function writes commands and queries to the instrument to modify

parameters and query device settings.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

writeBuffer

Variable Type       ViString

The user can use this control to send common commands and queries to

the instrument. This control can also be used to write any valid

command to the instrument.

Default Value:  "*RST"

Notes:

(1) The command or query to be sent to the instrument may be a

literal enclosed in double quotes i.e. "*RST" or may be contained in

a variable of type string.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.